NAVBOT-EN01

communication protocol

Contents

Revision History	3

BLE INFORMATION	4

DATA FRAME AGREEMENT	4

DATA FRAME DEMONSTRATION	5

Revision History

| DOCUMENT HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| SOFTWARE REVISION | CHANGES MADE |  | REVISION DATE | EDITED BY |
| 1.0 | Add BLE protocol |  | 5/1/2025 | XiaoleChen |

BLE INFORMATION

BLE NAME :

navbot_en01-******

SERVICE UUID:

6E400011-B5A3-F393-E0A9-E50E24DCCA9E

SEND DATA CHARACTERISTIC UUID:

6E400012-B5A3-F393-E0A9-E50E24DCCA9E

RECEIVE DATA CHARACTERISTIC UUID_TX:

6E400013-B5A3-F393-E0A9-E50E24DCCA9E

DATA FRAME AGREEMENT

| Byte | Name | Description |
| --- | --- | --- |
| Byte 1 | Header1 | Fixed 0x55 |
| Byte 2 | Header2 | Fixed 0xAA |
| Byte 3 | Null | Idle |
| Byte 4 | Null | Idle |
| Byte 5 | Null | Idle |
| Byte 6 | roll | Rolling Angle, value range -30 to 30. |
| Byte 7 | height | The height of the robot, with a value range of 32 to 85. |
| Byte 8 | linear_H | Linear speed, with a value range of -200 to 200. |
| Byte 9 | linear_L |  |
| Byte 10 | angular | The angular velocity of the robot, with a value range of -100 to 100. |
| Byte 11 | stable | The stable state of the robot: 1 on, 0 off |
| Byte 12 | mode | Fixed 0x01 |
| Byte 13 | dir | The movement direction of the robot is defaulted to 0, jump:1 |
| Byte 14 | joy_y | The Y-axis data of the joystick, with a value range of -100 to 100. |
| Byte 15 | joy_x | The X-axis data of the joystick, with a value range of -100 to 100. |
| Byte 16 | Null | Idle |
| Byte 17 | Null | Idle |
| Byte 18 | Null | Idle |
| Byte 19 | Null | Idle |
| Byte 20 | Null | Idle |

DATA FRAME DEMONSTRATION

55 AA 00 00 00 00 26 0000 00 01 00 00 00 00 00 00 00 00 00  // height = 0x26(38), stable = 1

After sending this instruction, the robot starts and the leg height is 38mm.

55 AA 00 00 00 00 30 0000 00 01 00 00 00 00 00 00 00 00 00  // height = 0x30(48), stable = 1 ，After sending this instruction, the robot starts and the leg height is 48mm.

55 AA 00 00 00 00 30 0000 00 00 00 00 00 00 00 00 00 00 00  // height = 0x26(38), stable = 0 ，After sending this instruction, the robot stops and the leg height is 48mm.

55 AA 00 00 00 00 26 0000 00 01 00 00 00 05 00 00 00 00 00  // height = 0x26(38), stable = 1 ，joy_y = 0x05(5) ,

After sending this instruction, the robot moves forward at a speed of 5.

55 AA 00 00 00 00 26 0000 00 01 00 00 00 FA 00 00 00 00 00  // height = 0x26(38), stable = 1 ，joy_y = 0xFA(-5)，

After sending this instruction, the robot moves backward at a speed of 5.

55 AA 00 00 00 00 26 0000 00 01 00 00 00 00 05 00 00 00 00  // height = 0x26(38), stable = 1 ，joy_x = 0x05(5) ,

After sending this instruction, the robot rotates clockwise with an angular velocity of 5.

55 AA 00 00 00 00 26 0000 00 01 00 00 00 00 FA 00 00 00 00  // height = 0x26(38), stable = 1 ，joy_x = 0xFA(-5)，

After sending this instruction, the robot rotates counterclockwise with an angular velocity of 5.

55 AA 00 00 00 00 26 0000 00 01 00 00 01 00 00 00 00 00 00  // height = 0x26(38), stable = 1 ，dir = 0x01，

sending this instruction, the robot jumps and is ready.

55 AA 00 00 00 00 26 0000 00 01 00 00 00 00 00 00 00 00 00  // height = 0x26(38), stable = 1 ，dir = 0x00，

After sending this instruction, the robot jumps in place.